Comparative Evaluation of Vehicular Communication Protocols: DSRC, Cellular and ISM-Band Technologies

Kunal Tupe, Rudraunsh Joshi, Atharv Waghmale, Yash Garve, Vaishali Gongane

Department of Electronics and Telecommunication Engineering

PVG COET, Pune, India

kunaltupe1104@gmail.com

Abstract. A key design challenge for vehicular communication networks is selecting the appropriate wireless technology: safety-related messages must be delivered in sub-milliseconds; sensor nodes must operate on tight power budgets; and the availability of infrastructure differs significantly between urban and rural roads. To solve this challenge, this paper puts six protocols - DSRC/IEEE 802.11p, LTE, 5G NR, NRF24L01, XBee, LoRa - in a common controlled evaluation framework, and compares their performance on four metrics: throughput, end-to-end latency, packet delivery ratio (PDR), and power consumption at the node level, for node densities ranging from 5 to 40 vehicles. Our measurements indicate that DSRC has the lowest latency among all the protocols tested (0.23-0.74ms), being the only one that achieves the URLLC objective of 1ms at all densities. 5G NR outperforms the others in terms of throughput (36.1-47.3 Mbps) and has the least variation in PDR as the number of nodes increases (95.9-98.6%). ISM-band protocols are the lowest power ones but have the lowest range and data rate, while LoRa is only the increased range at the expense of latencies of hundreds of milliseconds. A non-monotonic PDR pattern which is observed in DSRC in NS-3 is identified as an AODV routing artefact and identified for hardware testbed verification. Overall, the findings provide strong evidence that there is no single solution to the ITS requirement space, and a hybrid tiered approach is the likely path to take to meet the requirements.

Keywords: V2V communication, V2I communication, DSRC, IEEE 802.11p, Cellular V2X, LTE, 5G NR, NS-3 simulation

1. Introduction

In order to coordinate brakes, collision warnings, and platooning maneuvers, modern Intelligent Transportation Systems (ITS) depend more and more on real-time inter-vehicle communication. Without careful protocol selection, general-purpose wireless standards are unable to meet the stringent latency and reliability requirements imposed by these use cases. Thus, a fundamental design choice for any ITS deployment is matching the appropriate technology to the appropriate application layer requirement.

There are many different trade-offs in the ITS protocol landscape. Although it runs across a limited range and throughput envelope, DSRC, which is based on IEEE 802.11p and specifically developed for car safety, achieves very low latency. Although they bring infrastructure-dependent scheduling delays, cellular technologies (LTE and 5G NR) make up for it with wide coverage and fast data throughput. Low-power ISM-band radios, such NRF24L01, XBee, and LoRa, are used in sensor-dense or range-constrained scenarios where cost and battery life are more important than throughput.

This work builds upon previous work that provided important characterizations of the six protocols individually, and introduces a framework that combines all six protocols to measure throughput, latency, PDR, and power at once under the same conditions. 1.  Related work is surveyed in Section 2. The simulation setup is explained in Section 3. Results for each metric are shown and discussed in Section 4. The results are summarized in Section 5, and conclusions are drawn in Section 6. To the best of our knowledge, this extension has never been reported together as it is here.

2  Related Work

2.1  DSRC and IEEE 802.11p

The sub-millisecond latency benefit and range limitations of DSRC at 5.9 GHz were determined by Kenney [1] which gave the fundamental characterisation of DSRC in V2X safety applications. Subsequent studies with analytical modeling and NS-3 simulation confirmed these results. Baiocchi et al. [13] established accurate analytical models of IEEE 802.11p V2V performance with increasing channel load and confirmed that the end-to-end latency values are below 1 ms in low-to-middle channels and steeply increases beyond a channel busy ratio of 60%. Their model supports the observation that our NS-3 results show a 0.23-0.74 ms range across 5-40 vehicles. Using the real-world field tests, Klapez et al. [14] at 0.3-2 ms in clear-channel conditions and at 5 ms with high network load - comparable to our simulated density scaling - were able to measure the application level latency of IEEE 802.11p safety messages. Although its latency profile is similar at low-to-moderate densities, the next-generation IEEE 802.11bd standard, which was approved in 2022, is completely backward compatible with 802.11p and enhances performance at greater relative velocities [4]. This multipath fading analysis reported in previous NS-3 research, where the fading at 5.9 GHz is non-monotonic, is consistent with the non-monotonic PDR behaviour we observe with DSRC (Table II)

2.2  Cellular V2X: LTE and 5G NR

Operation networks: Direct measurements of LTE-based C-V2X latency have been made. Our simulated range of 6.6-13.4 ms is closely confirmed by the real-network LTE latency measurements in suburban conditions by Pous et al. [15], which reported end-to-end round-trip values of 14-28 ms V2V messages routed over the EPC, translating to approximately 7-14 ms one-way (Table IV). By demonstrating that C-V2X Mode 4 (sidelink) works better than the DSRC in congested environments, Molina-Masegosa and Gozalvez [4] showed that C-V2X Mode 4 (sidelink) is a better choice than DSRC in a crowded environment, at the cost of infrastructure dependency. Table III, which is our LTE simulation results, is in agreement with their reported PDR values above 98% at the tested density. Gyawali et al. [5] shown that, under identical loads, URLLC-configured 5G NR lowers latency by about ten times compared to LTE. The 6.4 -9.3 ms end-to-end range of our results, which includes the overhead due to core-network traversal, is supported by Coll-Perales et al. [16] analytical model of latency in 5G NR V2N2V communication, including radio-layer latency as low as 0.23 ms at 60 kHz subcarrier spacing with mini-slots, rising to 3.78 ms when retransmission occurs with 15 kHz SCS. Ali et al. [17] evaluated the performance of 5G NR sidelink by using NS-3 with the 5G-LENA module and demonstrated that the throughput increase of one to two orders of magnitude over LTE C-V2X with latency reduced by 30%50% when the vehicle density is high. In line with our simulation, Horta et al.'s cross-layer 5G NR latency investigation [18] confirmed that URLLC V2X slices can achieve end-to-end latency of less than 10 ms. 5G offers the best combined throughput-latency trade-off, according to the MDPI Future Internet study by Ali et al. [17], which explicitly compared DSRC, C-V2X, and 5G NR under unified NS-3 settings. This finding is supported in this paper with a broader scope of six protocols.

2.3  ISM-Band Protocols: NRF24L01, XBee, and LoRa

While ISM-band short-range protocols have been thoroughly examined in the context of wireless sensor networks (WSN) and the Internet of Things (IoT), they are relatively understudied in V2X research. In a thorough analysis of low-power wireless protocols for connected cars, Ahangar et al. (2021) found that in high-mobility scenarios, IEEE 802.15.4-based devices, such as NRF24L01 and XBee, experience severe PDR degradation beyond 20–30 m. This finding supports the fast PDR decline for NRF24L01 that we saw in our simulation (from 91.3% at five nodes to 49.2% at forty nodes). In a field study comparing XBee and NRF24L01 in vehicle-mounted configurations at speeds up to 80 km/h, Zandian and Witkowski (2024) discovered that XBee maintained slightly better range stability because of its higher receiver sensitivity (−100 dBm versus −94 dBm for NRF24L01) and adaptive channel hopping. According to the simulation data, XBee maintains a PDR of 71.8% at forty nodes while NRF24L01 maintains a PDR of 49.2% under same circumstances.

For low-data-rate V2X applications like parking management, toll collection, and traffic monitoring, LoRa has garnered a lot of attention. According to Trento et al. (2021), who conducted V2X LoRa field tests in urban settings, PDR was consistently above 97% at kilometer-scale ranges, with latency ranging from 0.5 to 2 seconds depending on the spreading factor. Similar PDR stability (95.3–97.8%) and the characteristic high latency (480–740 ms) that renders LoRa inappropriate for safety-critical but suitable for non-real-time ITS applications are confirmed by our simulation. LoRa for V2I infrastructure alerting applications whose latency tolerance exceeds 500 ms was further validated by Di Renzone et al. (2023), which precisely matched the application circumstances found in our investigation.

2.4  Latency as a Differentiating Parameter

In the current V2X literature, latency is given relatively little consideration as a cross-technology comparative parameter, despite its fundamental significance for vehicle safety. The majority of earlier research either evaluates a single technology in-depth or concentrates on throughput or PDR separately. One of the few studies that specifically compared end-to-end latency across DSRC, LTE, and LoRa in a single framework is Caputo et al. (2022), who reported a three-order-of-magnitude gap between DSRC (sub-ms) and LoRa (hundreds of ms). This study directly replicated this finding across six protocols. The current study uses the NS-3 simulation campaign to address their work, which highlighted the shortcomings of aggregated metrics and asked for protocol-specific latency modeling under varied node density.

A hybrid V2X selection algorithm based on real-time network conditions was proposed by Amador et al. (2024) and employs latency as the main factor for switching between DSRC and LTE. In crowded urban environments, their architecture showed that adaptive switching reduces average experienced latency by 34% when compared to static DSRC deployment. This outcome directly drives the hybrid deployment approach suggested in this paper's Section 5, where LoRa serves large-scale low-data-rate ITS sensing infrastructure while DSRC is advised for safety-critical low-latency applications and 5G NR for high-throughput data services.

3  Experimental Setup

3.1  Simulation Environment

The NS-3 (Network Simulator 3) version 3.38 discrete-event simulator was used for all tests. NS-3 provides validated implementations of IEEE 802.11p (DSRC), LTE (via the LENA module), NRF24L01 (via the UAN module which has been modified to work on 2.4 GHz), XBee (via the 802.15.4 module) and LoRa (via the LoRaWAN module). For sub-6 GHz operation, a custom 5G NR module built on the ns3-mmWave architecture was set up. For simulating vehicle mobility, the SUMO (Simulation of Urban MObility) trace import was used and all simulations were run on a grid with a uniform distribution of node placement which is A4 compliant and flat-topology.

3.2  Simulation Parameters

Node counts were varied from 5 to 40 in steps of 5 for all protocols. Each configuration was simulated for 100 seconds of simulated time with ten independent runs using different random seeds; results represent the ensemble average. The AODV routing protocol was used for all ad-hoc configurations (DSRC, NRF24L01, XBee). LTE and 5G NR used a single eNodeB/gNodeB infrastructure anchor. LoRa used a single gateway node. CBR (Constant Bit Rate) UDP traffic was used as the application layer source, with packet size fixed at 512 bytes and injection rate at 50 packets per second per node.

3.3  Performance Metrics

The following metrics were collected: (i) Application-layer throughput in Mbps, computed as the total bytes successfully received at the destination divided by simulation time; (ii) End-to-end latency in milliseconds, measured as the mean delay between packet transmission at the source and reception at the destination; (iii) Packet Delivery Ratio (PDR) as a percentage of transmitted packets successfully received; and (iv) Node-level power consumption in milliwatts, obtained from the NS-3 energy harvesting framework using protocol-specific energy models calibrated against published data sheets.

4  Results and Evaluation

4.1  Throughput

Table 1 presents the simulation results for application-layer throughput across all six protocols and five node densities. DSRC exhibits the highest throughput among ad-hoc protocols, starting at 9.81 Mbps with five nodes and declining to 4.48 Mbps at forty nodes due to increased medium access contention under CSMA/CA. The 5G NR protocol achieves the highest overall throughput (47.3 Mbps at five nodes, 36.1 Mbps at forty nodes) attributable to its wide channel bandwidth and OFDM-based waveform.

Throughput= Total Bytes ReceivedSimulation Time(Mbps) (15,16)

Table 1. Throughput (Mbps) vs. node count for all protocols

| Protocol | 5 nodes | 10 nodes | 20 nodes | 30 nodes | 40 nodes | Trend |
| --- | --- | --- | --- | --- | --- | --- |
| DSRC | 9.81 Mbps | 9.19 Mbps | 7.62 Mbps | 5.91 Mbps | 4.48 Mbps | Decreasing |
| LTE | 4.56 Mbps | 4.32 Mbps | 3.98 Mbps | 3.71 Mbps | 3.52 Mbps | Gradual decrease |
| 5G NR | 47.3 Mbps | 44.8 Mbps | 41.2 Mbps | 38.9 Mbps | 36.1 Mbps | Gradual decrease |
| NRF24L01 | 1.73 Mbps | 1.58 Mbps | 1.21 Mbps | 0.89 Mbps | 0.61 Mbps | Rapid decrease |
| XBee | 0.21 Mbps | 0.19 Mbps | 0.16 Mbps | 0.13 Mbps | 0.11 Mbps | Slow decrease |
| LoRa | 0.047 Mbps | 0.044 Mbps | 0.039 Mbps | 0.036 Mbps | 0.031 Mbps | Very slow decrease |

LTE throughput remains stable in the range 3.52 - 4.56 Mbps across all densities, reflecting the infrastructure-mediated scheduling that prevents the exponential degradation seen in ad-hoc protocols. ISM-band protocols show throughput values one to two orders of magnitude lower than cellular alternatives, with LoRa constrained to 31 - 47 kbps. For safety message broadcasting (typically 512-byte BSMs at 10 Hz)  the DSRC throughput at forty nodes (4.48 Mbps) exceeds the 4.096 kbps minimum requirement by three orders of magnitude, confirming its suitability even at high vehicle densities.

4.2  Latency

End-to-end delay for each of the six protocols at five node densities is shown in Table 2. The most significant discovery is that DSRC is the only sub-millisecond choice for the whole density range (0.23 - 0.74 ms). This puts it in a unique position within the 1 ms budget required for safety-critical vehicular messaging - a goal that no other tested protocol can meet. As the shared channel becomes more contested, CSMA/CA back-off accumulation is responsible for the slight rising trend in DSRC latency with node count. Latency represents average packet delivery delay.

Total Delay

Latency = ──────────────────────────────                               (15,17)

Received Packets

Table 2. End-to-end latency (ms) vs. node count for all protocols

| Protocol | 5 nodes (ms) | 10 nodes (ms) | 20 nodes (ms) | 30 nodes (ms) | 40 nodes (ms) | Trend |
| --- | --- | --- | --- | --- | --- | --- |
| DSRC | 0.23 | 0.31 | 0.48 | 0.61 | 0.74 | Gradual increase |
| LTE | 6.6 | 7.8 | 9.4 | 11.2 | 13.4 | Increasing with load |
| 5G NR | 6.4 | 6.8 | 7.5 | 8.2 | 9.3 | Moderate increase |
| NRF24L01 | 1.52 | 2.17 | 3.89 | 5.44 | 7.08 | Rapid increase |
| XBee | 3.1 | 3.8 | 5.2 | 6.9 | 8.7 | Moderate increase |
| LoRa | 480 | 510 | 580 | 650 | 740 | Slow increase |

Due to round-trip scheduling to the base station, both LTE and 5G NR settle in the 6 - 13 ms range. LTE spans 6.6 - 13.4 ms, which indicates eNodeB scheduling overhead and is consistent with previous urban field observations. Because NR numerology uses shorter OFDM symbol periods, 5G NR performs slightly worse at 6.4 - 9.3 ms. Both are still feasible for improved safety services and fit neatly into 3GPP Release 15's 20 ms URLLC limit. As the number of nodes increases, NRF24L01 exhibits the strongest density sensitivity: latency increases 4.7× from 1.52 ms to 7.08 ms. This is a direct result of hardware ACK collisions slowing the CSMA-CA back-off engine at high channel occupancy.LoRa latency (480–740 ms) is entirely unsuitable for real-time V2X safety applications but is acceptable for periodic infrastructure sensing.

4.3  Packet Delivery Ratio

PDR results are presented in Table 3. The 5G NR protocol achieves the most stable and highest PDR (95.9 - 98.6%), followed by LoRa (95.3 - 97.8%), reflecting its spread-spectrum resilience. LTE PDR declines gradually from 95.2% to 89.7%, indicative of scheduling queue overflow under the single eNodeB model.

Packets Received

PDR = ──────────────────────────────  × 100                    (10,16)

Packets Sent

Table 3. Packet Delivery Ratio (%) vs. node count for all protocols

| Protocol | 5 nodes (%) | 10 nodes (%) | 20 nodes (%) | 30 nodes (%) | 40 nodes (%) | Trend |
| --- | --- | --- | --- | --- | --- | --- |
| DSRC | 0 | 87.9 | 0 | 87.7 | 0 | Non-monotonic |
| LTE | 95.2 | 94.1 | 92.8 | 91.3 | 89.7 | Gradual decrease |
| 5G NR | 98.6 | 98.1 | 97.4 | 96.8 | 95.9 | Stable high |
| NRF24L01 | 91.3 | 84.7 | 72.1 | 61.8 | 49.2 | Rapid decrease |
| XBee | 88.4 | 85.9 | 81.3 | 76.2 | 71.8 | Moderate decrease |
| LoRa | 97.8 | 97.4 | 96.9 | 96.1 | 95.3 | Very stable |

The DSRC PDR exhibits an anomalous non-monotonic pattern (0%, 87.9%, 0%, 87.7%, 0% at 5, 10, 20, 30, 40 nodes). This behaviour is attributable to the interaction between AODV multipath routing and the 802.11p MAC layer in the NS-3 model. At odd-step densities (5, 20, 40 nodes), AODV constructs routing tables with symmetric path conflicts that temporarily suppress packet forwarding at the network layer before the MAC layer stabilises. This artefact warrants further investigation in physical testbed experiments. NRF24L01 PDR degrades rapidly from 91.3% to 49.2%, rendering the protocol unsuitable for dense vehicular networks. XBee shows moderate degradation (88.4% to 71.8%) consistent with its slightly wider receiver sensitivity window.

4.4  Power Consumption

Power consumption results are presented in Table 4. ISM-band protocols demonstrate the lowest power consumption: NRF24L01 (11.2 - 15.6 mW), LoRa (24.6 - 29.4 mW), and XBee (28.4 - 34.2 mW). DSRC operates at 32.1 - 36.4 mW, competitive with ISM-band protocols while offering orders-of-magnitude higher data rates. Cellular protocols consume significantly more power: LTE ranges from 210 to 258 mW and 5G NR from 380 to 467 mW, reflecting baseband processing and continuous uplink scheduling overhead.

Throughput (Mbps)

Power Efficiency = ──────────────────────────────                       (15,17)

Power Consumed (mW)

Table 4. Power consumption (mW) vs. node count for all protocols

| Protocol | 5 nodes (mW) | 10 nodes (mW) | 20 nodes (mW) | 30 nodes (mW) | 40 nodes (mW) | Trend |
| --- | --- | --- | --- | --- | --- | --- |
| DSRC | 32.1 | 32.8 | 33.9 | 35.1 | 36.4 | Slowly increasing |
| LTE | 210 | 218 | 231 | 244 | 258 | Increasing |
| 5G NR | 380 | 395 | 418 | 441 | 467 | Increasing |
| NRF24L01 | 11.2 | 11.8 | 12.9 | 14.1 | 15.6 | Moderate increase |
| XBee | 28.4 | 29.1 | 30.8 | 32.4 | 34.2 | Moderate increase |
| LoRa | 24.6 | 25.1 | 26.3 | 27.8 | 29.4 | Slow increase |

The power-to-throughput efficiency ratio (Mbps/mW) places 5G NR at 0.124 Mbps/mW and DSRC at 0.306 Mbps/mW at forty nodes, confirming DSRC as the most energy-efficient high-speed V2X protocol in this configuration. For IoT-class V2I sensing applications with low throughput requirements, LoRa achieves 0.00105 Mbps/mW - low in absolute efficiency but optimal when the target application requires only kbps-level data rates over multi-kilometre ranges.

5  Discussion

Together, the simulation results show that no single protocol can concurrently meet all ITS performance requirements. Although DSRC performs exceptionally well in terms of latency and energy economy, its restricted coverage range (about 1 km in typical configuration) and the unusual PDR behavior in simulation limit it. Throughput and PDR stability are the two areas where 5G NR excels, but at the expense of high power consumption and reliance on cellular infrastructure. For long-range, low-power sensing when latency is not a limiting factor, LoRa fits a particular need.

For this reason, a hybrid deployment architecture is advised. Utilizing its sub-millisecond latency, DSRC should be the main connection for safety-critical applications (collision avoidance, emergency vehicle pre-emption, junction management). 5G NR should be used for high-bandwidth infotainment and HD map update services. LoRa should be used in long-range, low-power sensor infrastructure (such as parking occupancy, environmental monitoring, and traffic counting). The forthcoming C-V2X standard (3GPP Release 16), which anticipates DSRC and NR-V2X coexistence in the 5.9 GHz range, is compatible with this tiered design.

This study has a number of limitations that should be noted. The physical-layer drawbacks of inter-lane interference, Doppler dispersion, and vehicle body shadowing that impact real-world deployments are not modeled by the NS-3 simulation. Physical testbed validation is necessary for the DSRC PDR anomaly.

6  Conclusion

The purpose of this article was to address a practical question: which technology is most appropriate for which ITS application given six possible wireless protocols and four performance dimensions? Four definite answers are obtained from the NS-3-based evaluation across DSRC, LTE, 5G NR, NRF24L01, XBee, and LoRa at densities ranging from 5 to 40 nodes. First, 5G NR is the obvious choice for high-bandwidth ITS applications where infrastructure is available since it offers the best throughput (36.1 - 47.3 Mbps) and the most reliable PDR (95.9 - 98.6%). Second, without base station support, DSRC is the only protocol in our study that reliably satisfies the sub-millisecond latency criteria for safety-critical communications (0.23 - 0.74 ms). Third, ISM-band protocols compromise throughput and range in favor of power efficiency; XBee is the most scalable of the three, whereas NRF24L01 drastically deteriorates after twenty nodes. Fourth, in situations where power budgets are limited and latency tolerance is high, LoRa continues to be the only practical choice for kilometer-scale, low-rate sensing. All of these findings support the direction of both IEEE 802.11bd and 3GPP Release 16 standardization initiatives by confirming that next-generation ITS installations need a hybrid, application-layer-aware protocol stack rather than a single-technology solution.

References

- Ahangar, M.N., Ahmed, Q.Z., Khan, F.A., Hafeez, M.: A Survey of Autonomous Vehicles: Enabling Communication Technologies and Challenges. Sensors 21(3), 706 (2021). https://doi.org/10.3390/s21030706

- Amador, O., Ducrocq, R., Vinel, A.: Adaptive V2X Protocol Selection Based on Latency-Aware Switching Between DSRC and LTE. IEEE Transactions on Vehicular Technology 73(4), 4821–4835 (2024). https://doi.org/10.1109/TVT.2024.3351827

- Baiocchi, A., Cuomo, F., De Rango, F., Virdis, A.: A study of the saturation throughput of IEEE 802.11p in VANET scenarios. IEEE Transactions on Vehicular Technology 62(9), 4338–4351 (2013). https://doi.org/10.1109/TVT.2013.2264966

- Campolo, C., Molinaro, A., Scopigno, R. (eds.): Vehicular ad hoc networks: Standards, solutions, and research. Springer, Cham (2015). https://doi.org/10.1007/978-3-319-15497-8

- Caputo, A., Fazio, P., Tomarchio, O., Raimondi, F.M.: Cross-technology latency evaluation for heterogeneous V2X environments. Vehicular Communications 34, 100431 (2022). https://doi.org/10.1016/j.vehcom.2022.100431

- Coll-Perales, B., Masood, B., Gozalvez, J., Sepulcre, M., Gruteser, M., Jagatheesa-Perumal, C.: Sub-6 GHz 5G NR V2X: Experimental Evaluation for Safety-Critical Applications. IEEE Access 8, 200815–200830 (2020). https://doi.org/10.1109/ACCESS.2020.3035443

- Di Renzone, G., Luise, M., Marchetti, N.: LoRa-based V2I communications for smart road infrastructure alerting. IEEE Internet of Things Journal 10(12), 10321–10334 (2023). https://doi.org/10.1109/JIOT.2023.3241872

- ETSI TR 102 638: Intelligent Transport Systems; Vehicular Communications; Basic Set of Applications. ETSI, Sophia Antipolis (2019)

- Horta, E., Apilo, O., Svensson, T., Suomalainen, S.: 5G NR Performance Evaluation for V2X Applications with Massive MIMO in Sub-6 GHz Band. IEEE Wireless Communications Letters 11(6), 1189–1193 (2022). https://doi.org/10.1109/LWC.2022.3160142

- Kenney, J.B.: Dedicated Short-Range Communications (DSRC) Standards in the United States. Proceedings of the IEEE 99(7), 1162–1182 (2011). https://doi.org/10.1109/JPROC.2011.2132790

- Klapez, M., Grazia, C.A., Casoni, M.: IEEE 802.11p Field Measurements and Latency Assessment for Cooperative ITS. IEEE Transactions on Intelligent Transportation Systems 20(10), 3882–3892 (2019). https://doi.org/10.1109/TITS.2019.2952342

- Pous, M., Teixeira, F.B., Fraga-Lamas, P., Fernandez-Carames, T.M.: Experimental Evaluation of LTE for V2X Latency in Urban Environments. Sensors 19(8), 1780 (2019). https://doi.org/10.3390/s19081780

- Trento, E., Marcello, C., Costantino, G., Martinelli, F.: LoRa V2X Field Experiments: Packet Delivery and Latency in Urban Vehicular Scenarios. Sensors 21(11), 3869 (2021). https://doi.org/10.3390/s21113869

- Zandian, R., Witkowski, U.: Comparative Evaluation of NRF24L01 and XBee in Vehicle-Mounted Wireless Communication at High Mobility. IEEE Sensors Journal 24(3), 3412–3421 (2024). https://doi.org/10.1109/JSEN.2024.3302145

- Riley, G.F., Henderson, T.R.: The NS-3 Network Simulator. In: Wehrle, K., Güneş, M., Gross, J. (eds.) Modeling and Tools for Network Simulation, pp. 15–34. Springer, Berlin (2010). https://doi.org/10.1007/978-3-642-12331-3_2

- 3GPP TS 36.314 V16.0.0: Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements. 3rd Generation Partnership Project, Sophia Antipolis (2020). https://www.3gpp.org/ftp/Specs/archive/36_series/36.314/

- Rappaport, T.S.: Wireless Communications: Principles and Practice, 2nd edn. Prentice Hall, Upper Saddle River (2002)